SETUP BEFORE WORKSHOP TO TEST:
Set-up arduino for wemos:

- Open Arduino and plug in wemos to computer using the USB cable

- Click Tools > Board > LOLIN(WEMOS) D1 R2 & mini

If this Board doesn’t show up on the list, follow these steps:

  - Click File > Preferences > Additional Board Managers

    - Paste the following into the box:

http://arduino.esp8266.com/stable/package_esp8266com_index.json

  - Click Tools > Boards > Boards Manager

    - Click Type > Contributed

    - Find esp8266 by ESP8266 Community and click install

  - Click Tools > Board > LOLIN(WEMOS) D1 R2 & mini

- Click Tools > Upload Speed > 115200

- Click Tools > Port > COMx, where x is any number greater than 1.

- Copy paste this code:

void setup() {

pinMode(BUILTIN_LED, OUTPUT); // set up the LED

}

void loop() {

digitalWrite(BUILTIN_LED, HIGH); // turn off LED

delay(500); // wait a half second

digitalWrite(BUILTIN_LED, LOW); // turn on LED

delay(500); // wait a half second

}

- Upload it to the wemos using this button: If it manages to upload you should see a light on the wemos flashing every second! The code is turning the light on for 0.5s then off for 0.5s. Everything within the { } brackets is looped / repeated. This is a good example code to show the students.

Code to talk about with the students:

void setup()  - function called at start will run only once

void loop() – everything within {} is looped consequitively after setup has finished

delay(x) – pause program for x number of milliseconds

Upcoming wiring:

Electronics session pt 1:

Start with the LED code above to demonstrate a loop and delays maybe?

MQ-135 air quality sensor. Data sheet

Talk about MQ-135 with the students:

- General “air quality” sensor – gasses: CO2, Smoke, NH3, NOx, Alcohol, Benzene

- Sensor has a layer of SnO2 - Tin Oxide which has a higher resistance / low conductivity in clear air. When there’s increase in polluting gasses, the resistance of sensor drops / conductivity increases and gives out a higher voltage reading.

- Analog vs digital – a range of values through wemos’ analog pin. Digital pins only have high and low.

- Can see a small spike by breathing on the sensor, much bigger spike by lighting a lighter near the sensor. I got over 100ppm for breathing and over 300ppm for lighter.

- Wire up MQ-135 -> wemos, using the analog pin. (left to right on the PCB back of MQ-135) Do on webcam if poss

  - A0 -> A0

  - Skip D0.

  - GND -> GND

  - VCC -> 5V

- Download MQ-135 library: https://github.com/ViliusKraujutis/MQ135 (click Clone or download > Download ZIP). in arduino click sketch >  library and add the zip

- add the following to the top of your code. Ask the students for a rough temperature and humidity values. Explain this is used to help calculate a ppm value for air quality.

#include "MQ135.h"

MQ135 mq135_sensor  = MQ135(A0);

float temperature = 21.0;

float humidity = 50.0;

- Below this add:

void setup(){

Serial.begin(115200);

}

void loop(){

float a = mq135_sensor.getCorrectedPPM(temperature, humidity);

Serial.print(a);

delay(5000);

}

- Upload the code, open serial monitor (make sure baud is 115200). The sensor takes ~60 seconds to heat up. Values should print from the sensor. Try breathing on the sensor to change the value – or even a lighter

Code to talk about with the students:

Serial.begin(x) - set the data rate x bits per second (baud)

Serial.print(x) - this prints x to the serial port, then readable on the serial monitor

Actual calculation happening (see more in the source library) based off datasheet

int val = analogRead(_pin);

return ((1023./(float)val) - 1.)*RLOAD;

return getResistance()/getCorrectionFactor(t, h);

Electronics session pt 2:

DHT11 temp / humidity sensor:

- Wire up DHT11 -> wemos; (left to right on DHT11):

  - S -> D7

  - Middle pin -> 3V3

  - - -> GND

- Download library zip: https://github.com/beegee-tokyo/DHTesp (click Clone or download > Download ZIP) in arduino click sketch >  library and add the zip

- add the following to the top of  your code:

#include "DHTesp.h"

DHTesp dht;

- Add the following to the loop() after “if (client.connect(server,80))”:

humidity = dht.getHumidity();

temperature = dht.getTemperature();

Serial.print("humidity: ");

Serial.print(humidity);

Serial.print(", temp: ");

Serial.println(temperature);

- Upload code and check serial monitor for temp & humidty readings. Tell the students this is now adjusting the ppm of the MQT-135 air quality sensor

Coding session

Sign in to Thingspeak:

- Thingspeak.com. Create an account as each one is limited to four channels.

- Click Channel > New Channel

  - Give it a name

  - Create channel

- Sharing > Share channel view with everyone (so students can view)

- API Keys > Write API Key. You’ll need this API key for the code below.

Create thingspeak/IoT test code

- Copy code from below. Zip of code here

- Fill in API Key, Wifi name & password

- Upload to wemos with button

- Once uploaded, click the serial monitor button

  - Change baud to 115200

  - You should see the wifi connect and the value get sent to teamspeak

  - Every 20 seconds it will send a new value to team speak (currently integer t which is incrementing by 1 every loop.

  - Have a look for your value on teamspeak coming in live!

  - Now you’ve created an IOT device, let’s add a sensor.

IoT Code:

#include <ESP8266WiFi.h>

// replace with your channel’s write API key (on the API Keys tab) and your wifi name and password

String writeApiKey = "write api key here";

const char* ssid = "wifi name here";

const char* password = "wifi password here";

const char* server = "api.thingspeak.com";

int t = 1;

WiFiClient client;

void setup(){

Serial.begin(115200);

delay(10);

WiFi.begin(ssid, password);

Serial.print("Connecting to ");

Serial.println(ssid);

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED){

delay(500);

Serial.print(".");

}

Serial.println("WiFi connected");

}

void loop()

{

if (client.connect(server,80)) {

String postStr = writeApiKey;

postStr +="&field1=";

postStr += String(t);

client.print("POST /update HTTP/1.1\n");

client.print("Host: api.thingspeak.com\n");

client.print("Connection: close\n");

client.print("X-THINGSPEAKAPIKEY: "+writeApiKey+"\n");

client.print("Content-Type: application/x-www-form-urlencoded\n");

client.print("Content-Length: ");

client.print(postStr.length());

client.print("\n\n");

client.print(postStr);

Serial.print("Value: ");

Serial.println(t);

Serial.println("Sending data to Thingspeak");

t = t + 1;

}

client.stop();

Serial.println("Waiting 20 secs");

// thingspeak needs at least a 15 sec delay between updates

// 20 seconds to be safe

delay(20000);

}

Code to talk about with the students:

char – store a character

string – store zero or more characters

Int t – store a number – in this case “a” you’re sending to thingspeak

a = a + 1  - incrementing integer a (within the loop)

Add the sensors to the IoT Code

Full Wemos to Thingspeak code with sensors etc￼

At the top:

#include "DHTesp.h"

DHTesp dht;

#include "MQ135.h"

MQ135 mq135_sensor  = MQ135(A0);

float temperature = 21.0;

float humidity = 50.0;

#include <SoftwareSerial.h>

At the end of setup:

dht.setup(D7, DHTesp::DHT11);

In void loop:

if (client.connect(server,80)) {

humidity = dht.getHumidity();

temperature = dht.getTemperature();

Serial.print("humidity: ");

Serial.print(humidity);

Serial.print(", temp: ");

Serial.println(temperature);

float a = mq135_sensor.getCorrectedPPM(temperature, humidity);

String postStr = writeApiKey;

postStr +="&field1=";

postStr += String(a);

postStr +="&field2=";

postStr += String(humidity);

postStr +="&field3=";

postStr += String(temperature);

Friday Matlab Session

Matlab to analyse data:

Go to https://matlab.mathworks.com/ and login with the same account from thingspeak.

Useful code:

[data,time] = thingSpeakRead(channelID, 'Numdays', x);

For reading the last number of days.
Replace channelID with your ID and with x number of days (can’t read more than 8000 points or ~3 days)

[data,time] = thingSpeakRead(channelID,'DateRange',[datetime('May 26, 2020'),datetime('Jun 1, 2020')])

For reading specific dates. Months only 3 letters.

plot(time,data)

To plot a graph of the sensors over time.

hold on

To keep this graph when adding more lines

avedata = movmean(data,x)

Getting the moving mean. Comparing to x number of previous and future points

plot(time,avedata)

Plot a graph of the moving mean.